ENSC 283

PROJECT

RUN OF THE RIVER

Nathan Zheng

301051095

Simon Fraser University

Mar 22nd, 2008

Introduction

Run-of-the-river hydroelectricity is a type of hydroelectric generation whereby the natural flow and elevation drop of a river are used to generate electricity. Power stations of this type are built on rivers with a consistent and steady flow, either natural or through the use of a reservoir at the head of the river. It has several advantages, for example, flooding the upper part of the river is not required as it doesn't need a large reservoir. As a result, people living at or near the river don't need to be relocated and natural habitats are preserved, reducing the environmental impact as compared to reservoirs. On the other hand, there exists certain disadvantage. The output of the power plant is highly dependent on natural run-off. Spring melts will create a lot of energy while dry seasons will create relatively little energy. Also, A run-of-the-river power plant has little or no capacity for energy storage and hence can't co-ordinate the output of electricity generation to match consumer demand.

In this project, we are going to explore the properties of fluid flow and its relation to the power generated by the turbine generator as one of the design parameter, namely, the diameter of the penstock, varies within a certain range.

Theory

The Run of Water project has typical design parameters as shown below:

To find the velocity of the fluid at the nozzle outlet, we need to apply the Bernoulli Equation. Here we assume the flow is steady incompressible, and neglect the friction head loss. The Bernoulli Equation then becomes:

p1 / ρ + V12 + gz1  =  p2 / ρ + V22 + gz2

Here V1 and V2 represent the velocities at the inlet and outlet of the penstock (including nozzle), and the other terms corresponds to pressure, density, gravity, vertical elevation, respectively.

Also, from mass flow conservation, we have the following relation:

V1A1 = V2A2  or  V1 = (A2 / A1) V2 = (dn2 / dp2)V2

Where dn represents the diameter of the nozzle and dp represents the diameter of the penstock.

From the above equations, we can express the velocity at the nozzle outlet as a function of the penstock diameter, if the other parameters are held constant:

V2  =  2 (p1 – p2) / ρ + 2g ( z1 – z2 )

1  –  dn4 / dp4

The kinetic and kinematic relations of the turbine can be illustrated by the following:

To calculate the power that can be generated by the turbine, we need to apply the momentum equation:

ΣFx = - Fb = muout  - muin

Where:

uin = V2  - Vb , uout = - (V2 – Vb ), Vb = ΩR

Note that here we use R to represent the sum of the radii of the turbine wheel and the bucket.

Combine all these equations, we obtain the expression for power generated per unit mass flow:

P / m = 2 (V2Vb – Vb2 )

Maximum value of this quantity occurs when its derivative with respect to the variable Vb is 0, that is:

d (P/m) / dt  = V2 -2Vb = 0, or Vb = V2 / 2

And the maximum power generated per unit mass flow is therefore:

P / m |max = V22 / 2

In addition, if we count the friction loss of the energy, we have the following formula,

hf = 0.316 (µ / ρVtd)1/4 LtVt2/ 2gd

where Vt is the average velocity of fluid in the tube and is therefore V1. d is the tube diameter and is therefore dp.

Results and discussions

The fixed design parameters are given as:

h = 200 m, L = 1000m, Rturbine = 0.8 m, Rbucket = 0.2 m, dn = 0.1 m

The fluid properties for water at 10oC are:

ρ = 1000 kg / m3 , γ = 9810 N / m3 , µ = 1.307E-3 Ns / m2

Based on these numerical values, we can obtain:

z1 = 26.35 m, z2 = 0, p1 = γz1 = 1704 kPa, p2 = pa = 101.3 kPa

Plug all these values into the nozzle jet velocity equation:

V2 = [3722 (1 – 0.0001 / dp4 ) ] 1/2

For dp varying from 0.2 to 1.5, the following results are obtained:

| d | v2 |
| --- | --- |
| 0.2 | 59.071 |
| 0.3 | 60.63 |
| 0.4 | 60.889 |
| 0.5 | 60.959 |
| 0.6 | 60.985 |
| 0.7 | 60.995 |
| 0.8 | 61.001 |
| 0.9 | 61.004 |
| 1 | 61.005 |
| 1.1 | 61.006 |
| 1.2 | 61.007 |
| 1.3 | 61.007 |
| 1.4 | 61.007 |
| 1.5 | 61.008 |

Based on V2, we can find the fluid velocity inside the penstock V1, the flow rate Q, the mass flow rate m, and the friction head loss hf. The values are tabulated below:

| d | v2 | v1 | Q | mdot | hf |
| --- | --- | --- | --- | --- | --- |
| 0.2 | 59.071 | 14.768 | 0.46394 | 463.94 | 1460.6 |
| 0.3 | 60.63 | 6.7367 | 0.47619 | 476.19 | 222.8 |
| 0.4 | 60.889 | 3.8056 | 0.47822 | 478.22 | 57.238 |
| 0.5 | 60.959 | 2.4384 | 0.47877 | 478.77 | 19.872 |
| 0.6 | 60.985 | 1.694 | 0.47897 | 478.97 | 8.3645 |
| 0.7 | 60.995 | 1.2448 | 0.47906 | 479.06 | 4.0233 |
| 0.8 | 61.001 | 0.95314 | 0.4791 | 479.1 | 2.1339 |
| 0.9 | 61.004 | 0.75313 | 0.47912 | 479.12 | 1.2197 |
| 1 | 61.005 | 0.61005 | 0.47913 | 479.13 | 0.73946 |
| 1.1 | 61.006 | 0.50418 | 0.47914 | 479.14 | 0.47023 |
| 1.2 | 61.007 | 0.42366 | 0.47915 | 479.15 | 0.31105 |
| 1.3 | 61.007 | 0.36099 | 0.47915 | 479.15 | 0.21267 |
| 1.4 | 61.007 | 0.31126 | 0.47915 | 479.15 | 0.14957 |
| 1.5 | 61.008 | 0.27114 | 0.47915 | 479.15 | 0.10777 |

We plot the flow rate Q and the friction head hf as a function of the penstock diameter:

Here we can see that the friction loss is negligible only for d > 0.4. Since we did our calculation of velocities based on negligible friction loss, we can say that our calculation for d < 0.4 is not accurate.

Now, if the turbine has an angular velocity of 300 rpm, that is, 31.4 rad/s, we can calculate the power generated based on the power equation developed previously. We tabulate the values and plot the graph.

| d | P |
| --- | --- |
| 0.2 | 8.06E+05 |
| 0.3 | 8.74E+05 |
| 0.4 | 8.86E+05 |
| 0.5 | 8.89E+05 |
| 0.6 | 8.90E+05 |
| 0.7 | 8.90E+05 |
| 0.8 | 8.91E+05 |
| 0.9 | 8.91E+05 |
| 1 | 8.91E+05 |
| 1.1 | 8.91E+05 |
| 1.2 | 8.91E+05 |
| 1.3 | 8.91E+05 |
| 1.4 | 8.91E+05 |
| 1.5 | 8.91E+05 |

The maximum power can also be calculated using previously developed formula:

| d | Pmax | Omax |
| --- | --- | --- |
| 0.2 | 8.09E+05 | 29.535 |
| 0.3 | 8.75E+05 | 30.315 |
| 0.4 | 8.86E+05 | 30.444 |
| 0.5 | 8.90E+05 | 30.48 |
| 0.6 | 8.91E+05 | 30.492 |
| 0.7 | 8.91E+05 | 30.498 |
| 0.8 | 8.91E+05 | 30.5 |
| 0.9 | 8.92E+05 | 30.502 |
| 1 | 8.92E+05 | 30.503 |
| 1.1 | 8.92E+05 | 30.503 |
| 1.2 | 8.92E+05 | 30.503 |
| 1.3 | 8.92E+05 | 30.504 |
| 1.4 | 8.92E+05 | 30.504 |
| 1.5 | 8.92E+05 | 30.504 |

We can notice that the max power and the power generated at 300 rpm (31.2 rad / s) do not vary by a big amount. We can see this is what we can expect by comparing the angular velocities corresponding to the max power with 31.2 rad / s, and finding that they are essentially the same.

Conclusion

As the penstock diameter increases, the power that can be generated by the Run of River power turbine will increase. However, for the penstock diameter > 0.5 m, the increment of power is not significant. The maximum power that can be generated by this turbine is about 0.9 MW, and it happened when the angular velocity of the turbine is around 300 rpm.

Comments

We did our calculations based on great amount of simplifications of the problem, for example, ignoring the friction head loss. This makes our calculation not accurate enough. If we include the friction head, we will end up with a relatively complicated non-linear equation when we calculate the nozzle outlet velocity. (see attached scratch paper). It requires more Matlab skills which we currently don’t possess.

Through the calculations, we found that the power generated by one turbine is not sufficient for a large-scale supply. Therefore more turbine generators are needed to be constructed. However, the cost of each turbine is not too bad comparing to the cost of a huge dam. In addition, the energy source is green and economical, and it will not cause too much environmental issues.

Appendix

Matlab script:

d = 0.2:0.1:1.5;

v2 = (3722.*(1 - 0.0001 ./ (d.^4))).^0.5;

v1 = (0.01 ./(d.^2)) .* v2;

Q = (pi./4).*0.01.*v2;

hf = (1.756.*v1.^(1.75) ).*d.^(-1.25);

mdot = 1000.*Q;

P = 2.*(31.4.*v2 -31.4^2 ).*mdot;

Pmax = (v2.^2).*mdot./2;

Omax = v2./2;

p1 = figure();

hold on;

title('velocity at jet exit');

plot(d,v2);

hold off;

%close(p1);

p2 = figure();

hold on;

title(' Flow Rate');

plot(d,Q);

hold off;

%close(p2);

p3 = figure();

hold on;

title ('friction head');

plot(d,hf);

hold off;

%close(p3);

p4 = figure();

hold on;

title ('Power at Omega = 300 rpm = 31.2 rad/s ');

plot(d,P);

hold off;

%close(p4);

p5 = figure();

hold on;

title ('Max power');

plot(d,Pmax);

hold off;

%close(p5);

p6 = figure();

hold on;

title ('Max omega');

plot(d,Omax);

hold off;

%close(p6);