Lab 3: Friction and Minor Losses in Pipelines

Alvin Ng

ENSC283

Written April 2009

Introduction

The purpose of this experiment is to introduce friction and minor losses in pipelines.  We do this by measuring flow losses and pressure characteristics of different pipes and piping components.  In this experiment we used PVC, copper, and galvanized steel pipes.  Other than material, turns, bends, contraction, expansion, slanted seat, membrane, and check valves also produce flow loss.  In this experiment, we studied the flow loss of sudden expansion and contraction of PVC pipes, uniform galvanized steel pipe, uniform copper pipe, and uniform PVC pipe.

Theory

For stead-state, incompressible, 1-dimensional flow, the energy equation is:

p1ρ+V122+gz1=p2ρ+V222+gz2+plossρ

where ploss is the pressure loss between sections 1 and 2.

There are two sources that causes pressure drop in pipelines.  The first is friction loss and wall shear stress.  The second is minor loss, which is caused from a change in geometry.

For fully-developed flow, friction pressure can be expressed by:

pfriction=ρfldV22

where d is channel diameter, l is channel length, f is friction factor, V is the average velocity.

Reynolds Number can be found by:

Re=ρVdμ

If the flow through the pipe is laminar, Re < 2300, friction factor can be expressed by:

f= 64Re

If the flow through the pipe is turbulent, the roughness of the pipe will be considered in part of the friction factor. The Reynolds number will then be Re < (65d/k). The range for Reynolds number will be 2320 < Re < 105.

f=0.31644Re

The friction factor for rough pipes could be found by:

f=-2log2.51Ref+0.27kd-2

To calculate the pressure for minor losses, we used:

pminor=ρζV22

where ζ is the coefficient of resistance.

ζ can be found using the following equation:

ζ=2plossρV2-fld

Data and Results

The volumetric flow rate, inlet pressure, outlet pressure, and temperature were measured on the apparatus we used.  We are going to use these measured values and some equations in the theory section to find the friction and minor losses in these pipelines.  The diameter, length, and roughness of the pipes were given.  The measured and calculated values could be found in the appendix.

1)

In a fully developed flow, there is one component where u = u(r). From this, we use the momentum equation for cylindrical coordinates, and this reduces to:

ρμδuδx=-dpdx+ρgx+1rδδrrτ

where τ can represent either laminar or turbulent shear. Since u = u(r), we can simplify the equation to:

1rδδrrτ=ddxp+ρgz

Since the LHS only varies with r and RHS only varies with x, we can assume that both sides must equal to the same constant. So now we integrate to get using τ = 0 @ r = 0

τ=12rddxp+ρgz=constr

For laminar flow:

τ=μdudr

Now we can write:

μdudr=12rK

where K=ddxp+ρgz. Then we integrate.

u=14r2Kμ+C1

To find C1, we use the initial condition u = 0 @ r = R. Then C1=-14R2Kμ. Now we can rewrite the equation to be:

ur=umax1-r2R2

The flow rate and velocity could be found easily after this:

Q=0Ru dA=πR48μ-ddxp+ρgz

V=12umax

Using the Darcy friction factor:

f=8τwρV2=88μVdρV2=64μρVd=64Re

Or this could be written as:

flaminar=64Red

2)

ζ=gd3hfLv2

3)

The flow regime affects the coefficient of resistance in expansion by decreasing the coefficient of resistance.  The flow regime affects the coefficient of resistance in constriction by increasing the coefficient of resistance.

Conclusion

This experiment was easy to do using the apparatus we used.  I bumped into a few problems with my calculations using the measured values.  I had no pressure drop for some of them, which doesn’t make sense.  I also had a very large number for pressure drop due to minor losses.  The pressure drop from minor loss was so great, it was large than the measured pressure drop.  I did not include the results of those calculations in my appendix because they were not significant since it just simply didn’t make any sense.

Appendix 1

Measurements

| Pipe Section Tipe | Diameter (mm) | Volumetric Flow Rate (L/min) | Inlet Pressure (mbar) | Outlet Pressure (mbar) | Temperature (°C) |
| --- | --- | --- | --- | --- | --- |
| Galvized steel, ½” | 16 | 25 | 446 | 403 | 29 |
| Galvized steel, ½” | 16 | 3.6 | 20 | 17 | 30 |
| PVC, 18x1 | 17 | 25.3 | 460 | 440 | 31 |
| PVC, 18x1 | 17 | 3.1 | 80 | 80 | 31 |
| Copper, 20x1.5 | 16 | 25.3 | 448 | 417 | 30 |
| Copper, 20x1.5 | 16 | 3.1 | 70 | 70 | 30 |
| Expansion | 17 to 28.6 | 25.8 | 470 | 470 | 31 |
| Expansion | 17 to 28.6 | 3.1 | 60 | 60 | 31 |
| Constriction | 17 to 14.5 | 24.3 | 510 | 460 | 31 |
| Constriction | 17 to 14.5 | 2.1 | 90 | 90 | 31 |

Appendix 2

Calculations

| Pipe Section Type | Volumetric Flow Rate (m³/s) | Reynolds Number | d/k | Regime |
| --- | --- | --- | --- | --- |
| Galvanized Steel, 1/2" | 0.000416667 | 40522 | 160 | Turbulent |
| Galvanized Steel, 1/2" | 0.00006 | 5958 | 160 | Laminar |
| PVC, 18x1 | 0.000421667 | 40251 | 17000 | Turbulent |
| PVC, 18x1 | 5.16667E-05 | 4932 | 17000 | Laminar |
| Copper, 20x1.5 | 0.000421667 | 41869 | 16000 | Turbulent |
| Copper, 20x1.5 | 5.16667E-05 | 5130 | 16000 | Laminar |
| Expansion | 0.00043 | 24399 | 28600 | Turbulent |
| Expansion | 5.16667E-05 | 2932 | 28600 | Laminar |
| Constriction | 0.000405 | 45326 | 14500 | Turbulent |
| Constriction | 0.000035 | 3917 | 14500 | Laminar |

| Pipe Section Type | Q(m^3/s) | f | dp(theoretical) | dp(experimental) | error% |
| --- | --- | --- | --- | --- | --- |
| Galvanized Steel, 1/2" | 0.00416667 | 0.034367 | 4602.805461 | 4300 | 6.5787152 |
| Galvanized Steel, 1/2" | 0.00006 | 0.042337 | 117.5831348 | 300 | 60.805622 |
| PVC, 18x1 | 0.000421667 | 0.0218994 | 2218.436233 | 2000 | 9.8464058 |
| PVC, 18x1 | 5.17E-05 | 0.0379262 | 57.68144269 | 0 | n/a |
| Copper, 20x1.5 | 0.000421667 | 0.0217162 | 2978.814577 | 3100 | 3.9092072 |
| Copper, 20x1.5 | 5.17E-05 | 0.0374796 | 77.18573893 | 0 | n/a |

| Fitting | Volumetric Flow Rate | f | Pressure Drop | Minor Pressure Drop | Coefficient of resistance (ζ) |
| --- | --- | --- | --- | --- | --- |
| Expansion | 0.00043 | 0.02456703 | 192.0349368 | n/a | n/a |
| Expansion | 5.17E-05 | 0.04469726 | 5.044200731 | n/a | n/a |
| Constriction | 0.000405 | 0.02135455 | 4420.599964 | n/a | n/a |
| Constriction | 0.000035 | 0.04073133 | 62.97170973 | n/a | n/a |

*Note n/a means the calculation couldn’t be done or the calculation was completely off, so it was not included in the table.