Lab 1: Center of Pressure and Hydrostatic Force on a submerged body

Alvin Ng

ENSC283

March 2009

Introduction

The purpose of this lab is to help understand hydrostatic pressure distribution.  We added weights on one end on the beam and we balanced the moment of that with water by adding water to the water vessel on the other side of the beam.  We calculated the distance to center of pressure from centroid, height of active surface, lever arm, and resultant using our measured results.

Theory

The height of active surface can be calculated using the equation:

h= s-stcos∝						(1)

The distance of the center of pressure from the planar centroid of the active surface is represented by:

e=s6 for s ≤ 100mm						(2)

e=(100mm)212(s-50mm) for s ≥ 100mm				(3)

To calculate the center of pressure, I used the governing equations:

ld=200mm-s3 , s ≤100mm					(4)

ld=150mm+e, s>100mm				(5)

To calculate Fp I used equation 6 and 7.

(6)

(7)

Fp=Pc∙Aact						(8)

These were the equations I used for my calculations.

Data and Results

| Angle, α, rad |  | Lowest water level, st, mm |  |  | Highest water level, sh, mm |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0.349066 |  | 15 |  |  | 109 |  |
| Lever arm, l, mm | Appended Weight, FG, N |  | Water level reading, s, mm | Calculated lever arm,ld,mm |  | Resultant Force, FP, N |
| 150 | 1 |  | 66 | 181.91 |  | 1.018 |
| 150 | 2.5 |  | 92.5 | 172.51 |  | 2.351 |
| 150 | 3 |  | 100 | 169.85 |  | 2.828 |
| 150 | 4 |  | 114 | 165.05 |  | 3.827 |
| 150 | 5.5 |  | 135.5 | 160.65 |  | 5.409 |
| 150 | 6.5 |  | 149 | 159.0 |  | 6.402 |
| 150 | 8.5 |  | 176 | 156.87 |  | 8.389 |

Table 1: Data set

Fig. 1: Center of pressure versus water level reading

Fig. 2: Resultant Force versus water level reading

Fig. 3: Water level reading versus appended weight

Discussion

The weight of the vessel and the beam is not included in the expression for the center of gravity because we balanced them.  The expression for the center of gravity will include the weight of the vessel and the beam, but it’ll just equal to zero, so it is pointless to even include them in the beginning.

The location of the center of pressure will not change if a different fluid were used in the tank because ρ is not used to calculate ld, the distance from the centre of the arc to the centre of pressure.

Conclusion

The lab was very straight forward and was completed successfully.  The lab helped me understand more about hydrostatic forces and the center of pressure by showing me the tilt angle of the vessel, water level reading, water level at the lowest point of the vessel, water level of active surface at rim, and height of active surface is related to find the location of center of pressure and the resultant force.