Lab 2: Energy Equations in a Venturi Tube

Alvin Ng

ENSC283

March 2009

# Introduction

# The purpose of our experiment was to introduce us to Bernoulli’s equation using the venturi tube. The venturi tube is an apparatus that is used for measuring the volumetric flow rate. It is also a good instrument to demonstrate Bernoulli’s equation because many of the variables from Bernoulli’s equation are known. We learned how to measure the piezometric head using the piezometer and the total head using a pitot tube. We later calculated the velocity of flow using two methods, the first method is using the total head, and the second method was using flow rate and the cross-section area of each position used to measure the piezometric head.

This lab has helped me understand and become more comfortable with Bernoulli’s equation.  I also learned the use and advantage of the venturi tube.

# Theory

The total energy along a streamline can be described using the Bernoulli’s equation:

Pρ+V22+gz=constant

The total energy head can be calculated by:

htot=Pρg+V22g+ z

The piezometric head can be calculated by:

hpiz=Pρg+ z

The velocity in the cross-section can be by using the total energy head and piezometric head equations.  By combining the last two equations:

V= 2g(htot-hpiz)

The velocity in the cross-section can also be described using volumetric flow rate. The velocity in the cross-section using flow rate is equal to:

V=QA

The flow rate factor of the venturi tube, K, can be determined from the flow rate and pressure loss between the largest and smallest diameter of the tube. It is described by:

K=Q∆P

In this experiment, I measured the pressure of the peizometer and pitot tube for two different values of flow rate, Q, using the venturi tube.  From these values, I calculated V and K using equation the velocity equations and K equation.  I calculated P using the piezometric head equation and I calculated Vhead using the total energy head equation and solving it for V2/2g.

# Data and Results

| Position | hpiz (mm) | htot (mm) | V (m/s) | K | hpiz* (mm) | htot* (mm) | V* (m/s) | K* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 200 | 214.5 | 0.5334 | 3.62E-06 | 176 | 193.5 | 0.5859 | 3.68E-06 |
| 2 | 189 | 215.5 | 0.7211 |  | 168 | 191 | 0.6718 |  |
| 3 | 37 | 213 | 1.8583 |  | 15 | 188 | 1.8424 |  |
| 4 | 126 | 206 | 1.2528 |  | 100 | 179 | 1.2450 |  |
| 5 | 144 | 188 | 0.9291 |  | 120 | 162.5 | 0.9132 |  |
| 6 | 152 | 180 | 0.7412 |  | 123 | 153 | 0.7672 |  |

Table 1: Data Set 1 and 2*

Fig. 1: Measured and calculated values of velocities versus length of venturi tube from Data 1

The measured velocity is the velocity calculated using hpiz and htot we measured.  The calculated velocity is the velocity found from using the measured flow rate and the cross-sectional area at each position.

Fig. 2: Velocity head versus the length of the venturi tube from Data 1

Fig. 3: Piezometric and total head versus length of the venturi tube from Data 1

Fig. 4: Measured and calculated values of velocities versus length of the venturi tube from Data 2*

Fig. 5: Velocity head versus the length of the venturi tube from Data 2*

Fig. 6: Piezometric and total head versus the length of the venturi tube from Data 2*

Discussion

On the head versus position graph, the slope of the total head curve seems to drop at position 4.  This drop is because the cross-sectional area at position 3 shrunk, and the energy was converted to a higher kinetic energy. At position 4, when the cross-sectional area increased again, the kinetic energy of the fluid was not completely transformed back into the total energy.  Some of the energy was lost into other sources of energy.  The slope is caused by a loss in energy in the system.  The assumption that leads to a horizontal total energy line is that energy is conserved.  The system is conserved.  But in our case, there was some energy loss, so the total energy line slopes downward a bit.

The flow of a fluid through a pipe is caused by a pressure difference.  The flow of the fluid travels from higher pressure to lower pressure.  The cross sectional area shrinks at position 3 and at that position, the velocity is greater because the area is smallest.  The increase in velocity causes an increase of kinetic energy which is then balanced by a drop in pressure so that energy is conserved.  The pressure cannot increase in the direction of uniform pipe flow because if it did, the flow will change directions.

Separation can be avoided by using larger diameter pipes for higher flow rates.  The larger diameter will help because it will prevent turbulent flow in the pipe.  The disadvantage of using a larger diameter pipe will be cost.

Cavitation is when bubbles are formed and these bubbles hit parts of the pipe at such high energy that it causes wear and tear and damages parts of the pipe.  The bubbles are created because of the pressure drops below vapor pressure.  Separation is when there is turbulence in the flow and eddy’s are formed and the flow isn’t smooth anymore.  This doesn’t damage parts of the pipe and no bubbles are formed.  The only thing that happens during separation is the flow becomes random and wouldn’t be nice like laminar flow.

Conclusion

This experiment has demonstrated that Bernoulli’s equation is true.  The venturi tube is a great way to demonstrate the Bernoulli’s equation, and the conservation of energy.  If the velocity of the flow increases, pressure decreases to keep a constant flow.  The flow rate factor from my experiment were similar because the measurements we took didn’t changed a lot, so the flow rate factor was almost the same. The lab was consistent with the theory we learned in class and helped me understand Bernoulli’s equation more.