Project: Run of the River

ENSC283

Alvin Ng

March 2009

Introduction

The run-of-the-river project is a type of hydroelectric generation whereby the natural flows of elevation drop of a river is used to generate electricity.  This method is a lot simpler to generate energy because there isn’t a need to re-direct flow of the river and build a large dam and flood the river to create an elevation drop to create energy.  The disadvantage is it’s not able to generate a large amount of electricity at once.

The run-of-the-river project is safer for the environment than a nuclear power plant because there will be no need to worry about a meltdown, a large amount of radiation releasing to the environment.  Nuclear power also creates nuclear waste which also emits dangerous radiation to the environment, and there is no safe way to dispose of it.  Also, nuclear reactors only operate for about forty to fifty years.

Coal and fossil fuel power plants require huge amount of fuel operate.  They release a lot of greenhouse gasses, mostly CO2, to the environment.  Coal and fossil fuel is also limited since it is difficult to transport them.  There is no need for coal specifically because none of our modern technology uses coal.  Since vehicles and people generally use a lot of fossil fuels, the earth is running low on fossil fuels, so keeping fossil fuel power plants will be pointless because we’ll run into a shortage of fossil fuel to burn.  The worst is still polluting the environment by releasing CO2 into the air.

Large dams require a lot of space and require changing the flow of water a bit to generate electricity.  Dams also require a reservoir and some flooding.  The run-of-the-river is friendlier to the environment because it doesn’t require a large reservoir, structures, or fuel to operate to generate electricity.  Since it doesn’t require fuel or nuclear power, the cost per kilowatt is much less than nuclear power or fossil fuels.  Compared to large dams, the structures are smaller, so cost of building one is easier.  The maintenance is less because all there is to maintain are the pipes and turbines.The main disadvantage of this project is its ability to store electricity.  Since the structures are so compact, there is no capacity to store and output electricity to match consumers accordingly.  Another main disadvantage is during dry season, this type of power plant will create much less electricity since it depends on spring melts.

In this project, we will look into the theory behind run-of-the-river.  We are going to see how the penstock, nozzle, and turbine generate electricity using the properties of fluid flow.

Theory

The design parameter of this project is shown in Fig.1.

Fig.1: Design parameters of run-of-the-river

Bernoulli’s equation of steady incompressible flow was used to solve for the velocity of the fluid at the nozzle.  I assumed there was no head loss.

p1ρ+v122+gz1=p2ρ+v222+gz2

where, P1 and P2 are pressure at the top and bottom of the penstock, V1 and V2 are velocity in and out of the penstock, ρ is density of the fluid, g is gravity, z is vertical elevation.

From the conservation of mass, flow rate in and out of the penstock must be equal. From this, we have the following equation:

Qin=Qout

Ainvin=Aoutvout

π4dp2vin=π4dn2vout

vin=dn2dp2vout

where Vin is the velocity of the fluid going into the penstock, Vout is the velocity of the fluid exiting the nozzle, dn is the diameter of the nozzle, and dp is the diameter of the penstock.

From the above equations, it is possible to express the velocity at the nozzle outlet as a function of the penstock diameter, when the other parameters are constant

v2=vout

v2=2(p1-p2)ρ+2g(z1-z2)1-dn4dp4

To calculate the power the turbine can generate, I used the momentum equation.  In this case the momentum equation is:

Fx=-Fb=muout-muin

where, uin=v2-vb, uout=-v2-vb, vb=ωr, r=R+b=0.8m+0.2m=1m

Substituting the previous equations and the momentum equation allows me to find power per unit mass flow rate:

Pm=2(v2vb-vb2)

To be able to find when a maximum occurs, I took the derivative of the power per unit mass flow rate with respect to Vb and equated it to 0 to find a critical point.

d(Pm)d(vb)=v2-2vb=0

vb=v22

From this, the maximum power per unit mass flow rate is:

Pm=v222

The friction head loss is given by:

hf=0.316μρvTd14LTvT22gd

where VT  is the average velocity of the tube, d is the diameter of the tube

In my project, I don’t know how to solve the implicit equation when I included the friction head loss into finding the velocity at the nozzle.  I decided to simplify my result by removing the friction head loss term, hf, to be able to calculate the velocity at the nozzle.

The MATLAB code is the equations I derived in this section. Then I plotted each parameter against dp.

Results and Discussion

The design parameters that are given are:

h=200m, L=1000m, R=0.8m, b=0.2m, dn=0.1m

The fluid properties of water at 5oC are:

ρ=1000kgm3, γ=9807Nm3, μ=1.519e-3N∙sm2

Based on these numerical values, we can calculate:

z1=26.35m, z2=0, p1=γ(h-z1)=1703kPa, p2=pa=101.3kPa

Plugging all these values we obtained into the velocity equation at the nozzle gives us:

v2=37241-0.0001dp4-112

I plugged all the equations into MATLAB and varied the diameter of the nozzle from 0.2 to 1.5.  This gave me Table 1.

| dp(m) | v2(m/s) | v1(m/s) | Q(m3/s) | m(kg·m/s) | hf(m) |
| --- | --- | --- | --- | --- | --- |
| 0.2 | 63.026 | 15.756 | 0.495 | 495 | 536.65 |
| 0.3 | 61.405 | 6.8228 | 0.48227 | 482.27 | 74.722 |
| 0.4 | 61.144 | 3.8215 | 0.48022 | 480.22 | 18.913 |
| 0.5 | 61.073 | 2.4429 | 0.47967 | 479.67 | 6.5397 |
| 0.6 | 61.048 | 1.6958 | 0.47947 | 479.47 | 2.7487 |
| 0.7 | 61.037 | 1.2457 | 0.47939 | 479.39 | 1.3213 |
| 0.8 | 61.032 | 0.95363 | 0.47934 | 479.34 | 0.7006 |
| 0.9 | 61.029 | 0.75345 | 0.47932 | 479.32 | 0.40037 |
| 1 | 61.028 | 0.61028 | 0.47931 | 479.31 | 0.24271 |
| 1.1 | 61.027 | 0.50435 | 0.4793 | 479.3 | 0.15434 |
| 1.2 | 61.026 | 0.42379 | 0.4793 | 479.3 | 0.10209 |
| 1.3 | 61.026 | 0.3611 | 0.47929 | 479.29 | 0.069797 |
| 1.4 | 61.025 | 0.31135 | 0.47929 | 479.29 | 0.049086 |
| 1.5 | 61.025 | 0.27122 | 0.47929 | 479.29 | 0.03537 |

Table 1: Calculated values of v2, v1, Q, m, and hf from varying dp

From Table 1, I plotted v2, Q, hf versus diameter of the penstock pipe,

Fig. 2: Velocity of fluid exiting nozzle versus diameter of pipe

Fig. 3: Flow rate out of the nozzle versus diameter of pipe

Fig.4: Friction head loss versus diameter of pipe

From the following data, it shows that as you increase the diameter of the penstock, the velocity at the nozzle decreases, and the flow rate decreases.  The flow rate decreases because as the diameter of the penstock increases, the velocity is dropping faster.  This would make sense that the velocity at the nozzle will drop, since flow rate is assumed to be constant initially. The pressure drop depends on whether the elevation changes when the diameter of the penstock is changed.  If the elevation z1 decreases, then the pressure will drop.

| dp(m) | P(kW) | Pmax(kW) | ωmax(rad/s) |
| --- | --- | --- | --- |
| 0.2 | 9.83E+05 | 9.83E+05 | 31.513 |
| 0.3 | 9.09E+05 | 9.09E+05 | 30.702 |
| 0.4 | 8.97E+05 | 8.98E+05 | 30.572 |
| 0.5 | 8.94E+05 | 8.95E+05 | 30.537 |
| 0.6 | 8.93E+05 | 8.93E+05 | 30.524 |
| 0.7 | 8.92E+05 | 8.93E+05 | 30.519 |
| 0.8 | 8.92E+05 | 8.93E+05 | 30.516 |
| 0.9 | 8.92E+05 | 8.93E+05 | 30.515 |
| 1 | 8.92E+05 | 8.93E+05 | 30.514 |
| 1.1 | 8.92E+05 | 8.93E+05 | 30.513 |
| 1.2 | 8.92E+05 | 8.93E+05 | 30.513 |
| 1.3 | 8.92E+05 | 8.92E+05 | 30.513 |
| 1.4 | 8.92E+05 | 8.92E+05 | 30.513 |
| 1.5 | 8.92E+05 | 8.92E+05 | 30.513 |

Table 2: Amount of power generated by the turbine for the speed of ω=300rpm and maximum power corresponding turbine speed

Fig. 5: Power versus diameter of pipe

The maximum speed of water at 5oC is 63.01m/s. The corresponding power, when the velocity of water at the nozzle is at maximum, is about 1MW.  This occurs when the turbine speed is 300rpm.

Fig. 6: Maximum power versus diameter of pipe

Fig. 7: Maximum angular velocity versus diameter of pipe

Maximum power and maximum turbine speed are directly related.  The generator can produce maximum power when the turbine speed is at maximum.  In this case, the diameter of the penstock will have to be 0.2m to produce maximum power.

Comments

From my data and calculations, a trend I saw is when the diameter of the penstock pipe is small, the flow rate is larger, velocity out of the nozzle is larger, which then spins the turbine faster and this would produce more power.  After enlarging the diameter so much, the flow rate will then level out to a pretty much constant flow, and velocity at the nozzle will do the same.  The whole system is determined by the flow going into the penstock.  The flow rate would decrease when the diameter of the penstock is increasing because the velocity of the fluid going into the penstock is dropping faster than the increase in diameter. I would recommend using a 0.5m or 0.6m diameter penstock pipe, because in reality, having everything operate at maximum speed will produce high wear and tear on parts and will be harder to maintain.  To eliminate high cost of using larger diameter pipes, the pipe that should be used will be the one when everything begins to level out which is roughly about 0.6m (2”) penstock pipe.  If the diameter of each pipe were to be really large, then an increase in elevation drop would help increase the inlet velocity, creating a larger flow rate.  It would be a good idea to put more turbines into this system because 0.9MW of energy is not enough to energy to power a whole city.  The size and cost of a turbine is probably still more economical than building a large dam even though a dam still has its advantages.

Unfortunately, my results are not accurate because I ignored the friction head loss when I did my calculations.  However, if I haven’t then the results I have obtained would be accurate enough to show that there are some losses in the system and everything isn’t linear.

Appendix

MATLAB code:

d = 0.2:0.1:1.5;

v2 = (3724./(1 - 0.0001 ./ (d.^4))).^0.5;

v1 = (0.01 ./(d.^2)) .* v2;

Q = (pi./4).*0.01.*v2;

hf = (0.576.*v1.^(1.75) ).*d.^(-1.25);

mdot = 1000.*Q;

P = 2.*(31.4.*v2 -31.4^2 ).*mdot;

Pmax = (v2.^2).*mdot./2;

Omax = v2./2;

p1 = figure();

hold on;

title('Velocity of Nozzle Varying Diameter of Penstock');

plot(d,v2);

hold off;

%close(p1);

p2 = figure();

hold on;

title('Flow Rate');

plot(d,Q);

hold off;

%close(p2);

p3 = figure();

hold on;

title ('Friction Head');

plot(d,hf);

hold off;

%close(p3);

p4 = figure();

hold on;

title ('Power at 31.4 rad/s');

plot(d,P);

hold off;

%close(p4);

p5 = figure();

hold on;

title ('Max Power');

plot(d,Pmax);

hold off;

%close(p5);

p6 = figure();

hold on;

title ('Max Omega');

plot(d,Omax);

hold off;

%close(p6);