List of Equipment:

- Lead Solder

- Soldering Iron/Station

- Third Hand

- Pliers

- Fume Fan

- Water and paper towel or something safe to use to clean the soldering iron tip

- A clean and safe space on the bench to work

- Eye/Face protection

- Gloves

- Wire cutters, if needed

Safety Precautions:

- Remove jewelry and accessories from hands and wrists

- Wear safety goggles/face shield

- Wear gloves

- Turn on the fume fan so that you don’t breathe in solder fumes

- Align the fume fan accordingly

- Always use the third hand when you are soldering

  - We don’t have workbench table surfaces; the soldering iron can easily damage the table

- Don’t solder directly below your face

  - Fumes will rise up and you will breathe in the harmful fumes

- The brass rod will get hot after soldering. Use pliers to hold the brass piece

To cut brass rod:

- Place the brass rod onto the table

- When making a cut, one should hold the smaller piece. This allows for complete control of the material you wish to use

To solder pieces of brass together:

- Place the brass rod onto third hand

- Apply heat with the soldering iron to the location on the brass in which you wish to connect the pieces

- Apply solder to the location in which you are heating, heat long enough for the solder to slightly run to assure that there are no air bubbles

- Repeat on the other piece

- Align both pieces to exactly where you wish to attach them

- Heat up the solder to connect the pieces, apply more solder if necessary

- When you are finished using the soldering iron, always clean the soldering tip and replace it back onto the holder

- Inspect the joint

- Repair/Redo if necessary

When you are finished using the soldering iron, coat the tip of the soldering iron with solder to prevent the tip from oxidizing.