List of Equipment:

- Lead Solder

- Soldering Iron/Station

- Third Hand

- Pliers

- Fume Fan

- Water and paper towel or something safe to use to clean the soldering iron tip

- A clean and safe space on the bench to work

- Eye/Face protection

- Gloves

- Wire cutters, if needed

Safety Precautions:

- Remove jewelry and accessories from hands and wrists

- Wear safety goggles/face shield

- Wear gloves

- Turn on the fume fan so that you don’t breathe in solder fumes

- Align the fume fan accordingly

- Always use the third hand when you are soldering

  - We don’t have workbench table surfaces; the soldering iron can easily damage the table

- Don’t solder directly below your face

  - Fumes will rise up and you will breathe in the harmful fumes

- The brass rod will get hot after soldering. Use pliers to hold the brass piece

To cut brass rod:

- Place the brass rod onto the table

- When making a cut, one should hold the smaller piece. This allows for complete control of the material you wish to use, and make sure that the pieces won’t fly out

To solder pieces of brass together:

- Place the brass rod onto third hand

- Apply heat with the soldering iron to the location on the brass in which you wish to connect the pieces

- Apply solder to the location in which you are heating, heat long enough for the solder to slightly run to assure that there are no air bubbles

- Repeat on the other piece

- Align both pieces to exactly where you wish to attach them together

- Heat up the solder to connect the pieces, apply more solder if necessary

- When you are finished using the soldering iron, always clean the soldering tip and replace it back onto the holder

- Inspect the joint

- Repair/Redo if necessary

When you are finished using the soldering iron, coat the tip of the soldering iron with solder to prevent the tip from oxidizing.